# DOC-01 Pre-Trip Inspection Checklist

Document ID: DOC-01 | Version 1.0 | Owner: Fleet Maintenance | Applies to: tractor units, straight trucks, light vans

Training document written for a portfolio project. Figures are illustrative and not official guidance.

## 1. Purpose and scope

Every driver completes a pre-trip inspection before moving a vehicle at the start of each shift. The inspection takes about 10 to 15 minutes and is recorded on the pre-trip log. Mechanics use the same checklist after any repair before returning a vehicle to service.

## 2. Walk-around checks

- Check that all lights and reflectors are clean, intact and working: headlights, brake lights, turn signals, hazard lights and marker lights.

- Look under the vehicle for fresh drips or puddles. Any leak is a defect (see DOC-05 for hydraulic fluid and DOC-09 for coolant).

- Check tires for cuts, bulges and low pressure (see DOC-04).

- Check wheels for missing or loose wheel nuts and rust streaks around the nuts.

- Check mirrors, windshield and wipers for damage.

## 3. Engine compartment checks

- Engine oil level is between the MIN and MAX marks on the dipstick.

- Coolant level is correct in the surge tank. Only check when the engine is cold (see DOC-09).

- Washer fluid is full.

- Drive belts are not cracked, frayed or loose.

- Battery terminals are tight and free from corrosion (see DOC-06).

## 4. Cab checks

- Seat belt, horn, gauges and warning lights work. No warning lights stay on after the engine starts.

- Steering has no unusual play.

- Parking brake holds when applied.

- Brake pedal feels firm and does not sink slowly to the floor.

## 5. Air brake test

- Start the engine and run it at fast idle. Air pressure must build from 85 psi to 100 psi within 45 seconds.

- With the engine off, fully apply the service brake and hold for one minute. Note any pressure drop (see DOC-02, section 5, for the allowed leak-down rate).

- Pump the brake pedal until the low-air warning buzzer or light comes on. It must come on before pressure falls below 60 psi.

- Drain air tanks daily if the vehicle does not have an automatic drain.

## 6. Completion and sign-off

Record the inspection on the pre-trip log. Report every defect using the Defect Reporting Procedure (DOC-07). If you find a Category 1 (critical) defect, do not move the vehicle. Tell your supervisor.