# DOC-02 Brake Inspection Procedure

Document ID: DOC-02 | Version 1.0 | Owner: Fleet Maintenance | Applies to: tractor units and trailers with air brakes

Training document written for a portfolio project. Figures are illustrative and not official guidance.

## 1. Purpose and scope

This procedure sets how often brakes are inspected and what the mechanic checks. Brakes are safety-critical. If you are unsure about any step, stop and ask your supervisor.

## 2. Inspection interval

- Tractor units: a full brake inspection every 6 weeks or every 10,000 km, whichever comes first.

- Trailers: a full brake inspection every 8 weeks.

- Any vehicle that has been in a collision, or reported a brake defect, gets an inspection before returning to service, whatever the schedule says.

- Drivers also check brakes at every pre-trip inspection (see DOC-01, section 5).

## 3. Lining and pad wear limits

- Disc brake pads: replace when friction material is less than 3 mm thick.

- Drum brake shoe linings: replace when friction material is less than 6 mm thick.

- Replace brake parts in axle sets (both wheel ends on the same axle), never one side only.

## 4. Drums and rotors

- Look for cracks, deep scoring and blue heat marks. A cracked drum or rotor is a Category 1 defect.

- Check for oil or grease contamination on linings. Contaminated linings are replaced and the leak source found.

- Lift the vehicle only following the lift and jack rules in DOC-10 before removing wheels.

## 5. Air system leak-down test

- Build the system to full pressure, then shut off the engine and release the parking brake.

- Wait one minute for pressure to settle, then fully apply the service brake.

- Allowed pressure drop with the brake applied: no more than 2 psi per minute for a single vehicle, and no more than 3 psi per minute for a tractor and trailer combination.

- A leak-down result above the limit is a Category 1 defect. Do not release the vehicle.

## 6. Slack adjusters and pushrod stroke

- Measure pushrod stroke with the brakes applied and compare to the limit marked on the brake chamber.

- Automatic slack adjusters should not need manual adjustment. If one is out of adjustment, treat that as a fault in the adjuster or its mounting. Do not just adjust it and move on.

## 7. Safety-critical rule

Any brake defect found is reported under DOC-07. For a Category 1 brake defect, apply a red tag, do not move the vehicle, and confirm the repair plan and return to service with a supervisor.

## 8. Records

Record the date, odometer reading, measured lining thickness, leak-down result and the mechanic’s name on the brake inspection sheet. Keep the sheet in the vehicle file.