# DOC-03 Oil and Filter Change Intervals

Document ID: DOC-03 | Version 1.0 | Owner: Fleet Maintenance | Applies to: all company vehicles

Training document written for a portfolio project. Figures are illustrative and not official guidance.

## 1. Purpose and scope

This document sets engine oil and filter change intervals, oil grades, the change procedure and how to dispose of used oil.

## 2. Change intervals

- Tractor units: every 25,000 km or every 12 months, whichever comes first.

- Light vehicles (vans and pickups): every 10,000 km or every 6 months, whichever comes first.

- Yard trucks and shunt tractors: every 500 engine hours or every 12 months, whichever comes first.

- Severe service (long idling, short trips, dusty sites): shorten the interval by 30 percent.

- The oil filter is always changed with the oil.

## 3. Oil grades

- Standard grade for the fleet is 15W-40 diesel engine oil.

- When regular temperatures are below -15 C, use the approved 5W-40 synthetic instead (see DOC-08).

- Never mix oil grades in one engine. Do not top up with a different grade unless it is an emergency, and then log it.

## 4. Change procedure

- Run the engine briefly to warm the oil, then shut it off.

- Place the drain pan under the sump and remove the drain plug. Let the oil drain fully.

- Remove the old oil filter. Check that the old gasket came off with it.

- Wipe the filter mount, lightly oil the new filter gasket and fit the new filter as the filter maker instructs.

- Refit the drain plug with a new washer. Tighten to the engine maker’s specification.

- Fill with the correct amount of new oil, then run the engine and check for leaks.

- Shut off, wait a few minutes, and recheck the dipstick level. Top up if needed.

## 5. Used oil and filter disposal

- Put used oil in the labelled used-oil drum. Never pour it down a drain or onto the ground.

- Drain used filters for at least 12 hours, then put them in the used-filter bin.

- Clean up spills at once with absorbent and place the used absorbent in the oily-waste bin.

## 6. Records

Update the service sticker in the cab door frame with the date and odometer or hour reading. Record the service in the maintenance log.