# DOC-04 Tire Pressure and Tread Checks

Document ID: DOC-04 | Version 1.0 | Owner: Fleet Maintenance | Applies to: all company vehicles

Training document written for a portfolio project. Figures are illustrative and not official guidance.

## 1. Purpose and scope

This document covers how to check tire pressure, tread depth and damage. Tires are checked at every pre-trip inspection (see DOC-01) and in detail at every service.

## 2. Pressure checks

- Check pressure when tires are cold, meaning the vehicle has been parked for at least 3 hours or driven less than 2 km.

- Use a calibrated gauge. Do not judge pressure by looking or by kicking the tire.

- The correct pressure is on the tire placard on the vehicle. If the placard is missing or unreadable, report it as a defect under DOC-07. Do not guess a pressure.

- On dual wheels, the two tires must be within 5 psi of each other.

- Pressure falls in cold weather, so check more often in winter (see DOC-08).

## 3. Tread depth limits

- Steer axle tires: replace at 3 mm tread depth or less (about 4/32 inch).

- Drive and trailer axle tires: replace at 1.6 mm tread depth or less (about 2/32 inch).

- Measure in the main grooves with a tread depth gauge, at three points across the tire.

## 4. Damage checks

- Look for cuts, bulges, exposed cord, sidewall cracks and uneven wear.

- Check between dual tires for stones or debris.

- Exposed cord, a bulge, or a sidewall cut that reaches the cords is a Category 1 defect. Do not move the vehicle.

## 5. Wheel nuts

Check wheel nuts for looseness and for rust trails or shiny marks around them. Loose-nut indicators, if fitted, should still point in line. Report any loose or missing nut as a Category 1 defect.

## 6. Flat or very low tires

- If a tire is flat or has lost more than 20 percent of its target pressure, do not inflate it. A tire that has run flat can fail violently.

- Move away from the tire, keep others clear, and call your supervisor. Re-inflation is done only by a trained person using a safety cage and a clip-on chuck with a remote hose.

## 7. Tire changes

Wheel removal follows the lift and jack rules in DOC-10. Wheel nut tightening values come from the vehicle maker’s manual.