# DOC-05 Hydraulic Leak Response

Document ID: DOC-05 | Version 1.0 | Owner: Fleet Maintenance | Applies to: trucks, trailers with hydraulic systems, lift gates, yard equipment

Training document written for a portfolio project. Figures are illustrative and not official guidance.

## 1. Purpose and scope

This document explains what to do when you find or suspect a hydraulic fluid leak. Hydraulic systems run at very high pressure and a small leak can cause serious injury.

## 2. Hazards

- Fluid escaping from a pinhole can be forced through skin. Never check for a leak with your hand, fingers or a rag held in your hand.

- Hot fluid causes burns, and spilled fluid creates slip and fire hazards.

- Use a piece of cardboard or wood to find a suspected leak, and wear safety glasses and gloves.

## 3. Immediate response

- Stop work and stop the machine or vehicle in a safe place.

- Lower any raised lift gate, bucket, ramp or load to the ground or onto supports.

- Shut off the engine and remove the key.

- Wait for the system to cool and let pressure bleed off before you go near the leak.

- Keep other people away from the area.

## 4. Classify the leak

- Class A, weep or seep: damp or wet fittings with no drip. Record it and report it at the end of the shift. Check the fluid level.

- Class B, drip: fluid drips from the component. Report it, and do not use the equipment again until a mechanic has inspected it.

- Class C, running or spraying: fluid runs, streams or sprays. Do not operate the equipment. Tag it out, use the spill kit, and call your supervisor straight away.

## 5. Containment and clean-up

- Put absorbent pads or granules around and under the leak to stop it spreading.

- Block drains and keep fluid away from storm drains and waterways.

- Place used absorbent in the oily-waste bin (see DOC-03, section 5).

- Tell your supervisor about any spill larger than 5 litres so it can be reported to the environmental officer.

## 6. Injection injury first aid

If fluid has penetrated the skin, even if the wound looks small or painless, treat it as a medical emergency. Call emergency services or go to hospital at once. Tell the doctor that it was high-pressure hydraulic fluid. Do not wipe, squeeze or try to treat the wound yourself.

## 7. Reporting

Every hydraulic leak is reported under the Defect Reporting Procedure (DOC-07). Class C leaks are Category 1 defects. Confirm the return-to-service plan with a supervisor.