# DOC-08 Winter Operation Checklist

Document ID: DOC-08 | Version 1.0 | Owner: Fleet Maintenance | Applies to: all vehicles, from 1 November to 31 March or whenever temperatures fall below 0 C

Training document written for a portfolio project. Figures are illustrative and not official guidance.

## 1. Purpose and scope

Cold weather affects starting, fuel, batteries, brakes and tires. This checklist adds winter steps to the normal pre-trip inspection (DOC-01).

## 2. Cold starts and block heaters

- When the temperature is below -10 C, plug in the engine block heater at least 2 hours before the planned start.

- Do not crank the engine for more than 15 seconds at a time. Wait 2 minutes between attempts.

- Let the engine idle and warm up before driving off. Do not rev a cold engine.

## 3. Fuel

- Use winter-blend diesel, or add anti-gel additive, when temperatures are below -10 C.

- Drain the fuel-water separator every day in winter.

- Keep fuel tanks at least half full to limit condensation.

## 4. Engine oil

Use the approved 5W-40 synthetic when regular temperatures are below -15 C. Grades and intervals are in DOC-03.

## 5. Batteries

- Have batteries load-tested before winter. Cold reduces battery power sharply.

- If a vehicle will not start, follow DOC-06 for the jump-start steps. Never jump-start a frozen battery.

## 6. Air brakes

- Drain air tanks every day. Water in the system can freeze and block brake valves.

- Check the air dryer and replace the cartridge at the scheduled interval.

- Do not use the parking brake on a wet, freezing vehicle left standing for long periods if it can be avoided. Use wheel chocks.

## 7. Tires

Tire pressure falls about 1 psi for every 5 C drop in temperature, so check pressures at least once a week and at every pre-trip inspection (see DOC-04).

## 8. Lights, glass and snow

- Clean all lights, mirrors and glass before moving.

- Use washer fluid rated to -40 C.

- Clear snow and ice from the roof, hood and trailer top before driving off.

## 9. Winter emergency kit

Each vehicle carries a warm blanket, flashlight with spare batteries, reflective triangles, tire chains where required, a shovel, and a high-visibility vest.