# DOC-09 Coolant and Cooling System Checks

Document ID: DOC-09 | Version 1.0 | Owner: Fleet Maintenance | Applies to: all diesel and gasoline vehicles

Training document written for a portfolio project. Figures are illustrative and not official guidance.

## 1. Purpose and scope

This document explains coolant level checks, the coolant mix, hose and belt checks, and what to do if an engine overheats.

## 2. Checking the coolant level

- Check the level only when the engine is cold. Look at the level marks on the surge tank.

- Never open the radiator or surge tank cap on a hot engine. Hot coolant under pressure can scald you badly.

- A coolant level that keeps dropping is a defect. Report it under DOC-07.

## 3. Coolant type and mix

- Use the approved extended-life coolant mixed 50/50 with distilled water.

- Never mix different coolant types or colours.

- Test the freeze protection with a refractometer every 6 months and before winter. The mix should protect to at least -37 C.

## 4. Hoses, belts and radiator

- Squeeze hoses when the engine is cold. Replace hoses that are cracked, bulging or soft.

- Check belts for cracks and fraying and for correct tension.

- Clear dirt, leaves and insects from the front of the radiator and the charge-air cooler.

## 5. Overheating response

- If the temperature gauge enters the red zone or a warning light comes on, pull over in a safe place as soon as you can.

- Let the engine idle for 2 to 3 minutes to cool, then shut it off. Switch on hazard lights.

- Do not open the cap. Wait at least 30 minutes for the engine to cool.

- Do not restart until the cause is found. Call your supervisor and report a defect under DOC-07.

## 6. Service interval

Change coolant every 3 years or 600,000 km, whichever comes first, and flush the system at the same time.

## 7. Used coolant disposal

Used coolant is hazardous waste. Put it in the labelled used-coolant drum. Never pour it onto the ground or down a drain. Clean up spills at once with absorbent.