# DOC-10 Lift, Jack and Lockout Safety

Document ID: DOC-10 | Version 1.0 | Owner: Fleet Maintenance and Health and Safety | Applies to: all work under or around raised vehicles

Training document written for a portfolio project. Figures are illustrative and not official guidance.

## 1. Purpose and scope

This document covers safe lifting of vehicles and lockout before working on stored-energy systems. Working under a vehicle is one of the most dangerous jobs in the garage.

## 2. Never rely on a jack alone

- Never get under a vehicle that is held up by a jack only. Jacks are for lifting, not for holding.

- Support the vehicle on rated jack stands or use a vehicle hoist with its mechanical locks engaged.

- Check the rating of the stand against the vehicle’s weight, including any load.

## 3. Setting up

- Work on firm, level ground, such as a concrete floor. Never lift on soft ground, gravel or ice.

- Set the parking brake, put the transmission in park or neutral as appropriate, and chock the wheels that stay on the ground.

- Use only the maker’s lifting points shown in the vehicle manual.

- Keep other people clear while lifting and lowering.

## 4. Hoists and lifts

- Inspect the hoist before each shift for damage and leaks.

- Never exceed the rated capacity. Do not use a hoist with a damaged lock or a leaking cylinder. Tag it out.

## 5. Lockout and tagout

Before working on a system that can store energy (air brakes, hydraulics, battery circuits, suspension), lock out and tag out:

- Shut off the engine and remove the key. Keep the key with you.

- Disconnect the battery negative cable if you will work on electrical circuits.

- Bleed off stored air or hydraulic pressure and secure any raised parts (see DOC-05).

- Attach your personal lock and tag. Only the person who put on a lock may remove it.

## 6. Wheel removal

Before wheel removal, the vehicle must be properly supported under sections 2 and 3. Break loose wheel nuts while the wheel is still on the ground.

## 7. Supervisor confirmation

For any job where you are not sure the vehicle is secure, stop and confirm with a supervisor before going under it. This is a safety-critical rule.